# 3,500 ft Rocket — Electronics & Motor Sourcing

Reference sheet for Step 1 (OpenRocket modeling) and Step 2 (ordering) of the project plan.

# Flight Computer Electronics

Generic/clone parts — cheap and well-documented, matching the frugal DIY approach in the project plan.

| Function | Part | Why This One / Notes | Est. Cost |
| --- | --- | --- | --- |
| Microcontroller | Arduino Nano (clone) | Small, breadboard-friendly, huge library support | $3–6 |
| Altitude | BMP388 barometric sensor | Better resolution & faster sampling than BMP280 | $3–7 |
| Accel / gyro | MPU6050 | Cheapest common 6-axis IMU, well-supported libraries | $2–5 |
| Position | NEO-6M GPS module | See COCOM limit warning below before buying | $8–15 |
| Storage | MicroSD card module (SPI) + microSD card | Backup log if radio link drops out mid-flight | $3–6 |
| Radio | 2× RFM95W LoRa module (915 MHz) | One for rocket, one for ground station | $8–15 each |
| Power | 1S LiPo (500–900mAh) + buck/boost regulator | Match regulator output to your sensors' voltage | $6–12 |

# GPS Warning — Read Before Buying

Cheap GPS modules like the NEO-6M often ship with a built-in altitude/speed limit (commonly around 1,000–1,500 m altitude and/or 500 m/s velocity), due to COCOM export restrictions meant to prevent missile-guidance use.

3,500 ft is about 1,067 m — right at that edge — and G-motor rockets can reach speeds where the velocity limit matters too. Before purchasing, check the listing for “COCOM limits” or look specifically for a module advertised as unlocked / high-altitude. Otherwise the GPS may simply stop reporting mid-flight.

# Motor Sourcing

No NAR/Tripoli certification is required for G-class motors — they're sold over the counter as standard hobby items.

| Source | Notes |
| --- | --- |
| Apogee Rockets | Carries 29mm AeroTech / Cesaroni G motors, 100–150 Ns range. Also body tubes, nose cones, fin stock. |
| Wildman Rocketry | Good for price comparison on the same motor classes and airframe parts. |
| Local hobby shop | Worth a call — shops that stock rocketry (not just RC cars) may carry Estes/AeroTech G motors, saving on shipping. |
| ThrustCurve.org | Not a vendor — use this to pull exact thrust-curve data for whichever motor you buy, to import into OpenRocket. |

# Motor Selection Guidance

- Target 29mm motor mount

- Look for AeroTech or Cesaroni G-series motors in the 100–150 Ns total impulse range (e.g. AeroTech G74, G77)

- Wait to purchase the motor until close to your actual launch date — no reason to store propellant for weeks

Vendor links:

Apogee Rockets

Wildman Rocketry

ThrustCurve.org